Aim: Algorithm to implement data set of characters the three CRC polynomials – CRC 12, CRC 16 and CRC CCIP.

Algorithm

- Input the data (binary digits).

- Choose the generator polynomial depending on the CRC type.

- Append the required number of zero bits (equal to degree of polynomial-1) to the end of data.

- Perform binary division (modulo-2) using XOR.

- The remainder after division is the CRC checksum.

- Append the CRC checksum to the original binary data to get the final codeword.

Cyclic Redundancy Check for 12 bit Polynomial

Cyclic Redundancy Check for 16 bit Polynomial

Cyclic Redundancy Check for Consultative Committee for International Telegraphy and Telephony 16 bit Polynomial

PROGRAM:

#include <stdio.h>

#include <string.h>

void xorOperation(char *dividend, char *divisor, int length) 	#XOR Operation

{

for (int i = 1; i < length; i++) {

dividend[i - 1] = ((dividend[i] - '0') ^ (divisor[i] - '0')) + '0';

}

}

// Perform Modulo-2 division

void computeCRC(char *data, char *poly, char *remainder, int polyLen) {

int dataLen = strlen(data);						#XOR division

char temp[128];

strncpy(temp, data, polyLen);

temp[polyLen] = '\0';

for (int i = polyLen; i <= dataLen; i++) {

if (temp[0] == '1')

xorOperation(temp, poly, polyLen);

else

xorOperation(temp, "00000000000000000", polyLen); // Dummy zero divisor

temp[polyLen - 1] = (i < dataLen) ? data[i] : '0';

}

strncpy(remainder, temp, polyLen - 1);			#polyLen – 1 is CRC checksum

remainder[polyLen - 1] = '\0';

}

int main() {

char binData[128];

printf("Enter binary data: ");

scanf("%s", binData);

char dataPadded[160], remainder[64];

// CRC-12

char crc12[] = "1100000001111";  // degree 12

strcpy(dataPadded, binData);

strcat(dataPadded, "00000000000"); // append 11 zeros

computeCRC(dataPadded, crc12, remainder, 12);

printf("CRC-12 Checksum: %s\n", remainder);

// CRC-16

char crc16[] = "11000000000000101"; // degree 16

strcpy(dataPadded, binData);

strcat(dataPadded, "000000000000000"); // append 15 zeros

computeCRC(dataPadded, crc16, remainder, 16);

printf("CRC-16 Checksum: %s\n", remainder);

// CRC-CCITT

char crcCCITT[] = "10001000000100001"; // degree 16

strcpy(dataPadded, binData);

strcat(dataPadded, "000000000000000"); // append 15 zeros

computeCRC(dataPadded, crcCCITT, remainder, 16);

printf("CRC-CCITT Checksum: %s\n", remainder);

return 0;

}

OUTPUT