Experiment 6.

AIM: Write a C Program to implement Sliding window protocol for Goback N.

PROGRAM:

#include <stdio.h>

#include <stdlib.h>

#include <time.h>

#define WINDOW_SIZE 4 // Example window size

void sender(int total_frames) {

int frames_sent = 0;

int next_frame_to_send = 0;

int ack_expected = 0;

printf("Sender: Starting transmission...\n");

while (ack_expected < total_frames) {

// Send frames within the window

for (int i = 0; i < WINDOW_SIZE && (next_frame_to_send < total_frames); i++) {

printf("Sender: Sending Frame %d\n", next_frame_to_send);

frames_sent++;

next_frame_to_send++;

}

// Simulate acknowledgment or timeout

// For simplicity, we'll assume a random success/failure for the first frame in the window

// In a real scenario, this involves timers and actual network communication

if (rand() % 5 != 0) { // Simulate successful ACK for the first frame in the window

printf("Receiver: Acknowledgment received for Frame %d\n", ack_expected);

ack_expected++;

} else { // Simulate timeout or lost ACK

printf("Sender: Timeout! Retransmitting from Frame %d\n", ack_expected);

next_frame_to_send = ack_expected; // Go back N

}

printf("\n");

}

printf("Sender: All frames transmitted successfully.\n");

}

int main() {

srand(time(NULL)); // Seed for random number generation

int total_frames;

printf("Enter the total number of frames to send: ");

scanf("%d", &total_frames);

sender(total_frames);

return 0;

}

OUTPUT

Experiment 7.

AIM: Write a C Program to implement Sliding window protocol for Selective repeat.

PROGRAM:

#include <stdio.h>

#include <stdlib.h>

#include <time.h>

#define WINDOW_SIZE 4 // Example window size

void sender_sr(int total_frames) {

int next_frame_to_send = 0;

int base = 0; // The oldest unacknowledged frame

int sent_frames[total_frames]; // To keep track of sent frames

// Initialize sent_frames array

for (int i = 0; i < total_frames; i++) {

sent_frames[i] = 0; // 0 means not yet sent/acked

}

printf("Sender: Starting Selective Repeat transmission...\n");

while (base < total_frames) {

// Send frames within the window

for (int i = 0; i < WINDOW_SIZE && (next_frame_to_send < total_frames); i++) {

if (sent_frames[next_frame_to_send] == 0) { // Only send if not already sent/acked

printf("Sender: Sending Frame %d\n", next_frame_to_send);

sent_frames[next_frame_to_send] = 1; // Mark as sent

}

next_frame_to_send++;

}

// Simulate acknowledgments and potential retransmissions

// For simplicity, we'll randomly simulate ACKs for frames within the window

// In a real scenario, this involves timers and actual network communication

for (int i = base; i < next_frame_to_send && i < total_frames; i++) {

if (sent_frames[i] == 1) { // If frame was sent and not yet acknowledged

if (rand() % 3 != 0) { // Simulate successful ACK

printf("Receiver: Acknowledgment received for Frame %d\n", i);

sent_frames[i] = 2; // Mark as acknowledged

} else { // Simulate lost ACK or damaged frame, requiring retransmission

printf("Sender: Timeout/NAK for Frame %d. Retransmitting...\n", i);

sent_frames[i] = 0; // Mark for retransmission

}

}

}

// Slide the window forward if frames at the base are acknowledged

while (base < total_frames && sent_frames[base] == 2) {

base++;

}

printf("\n");

// Reset next_frame_to_send to base to re-evaluate sending opportunities

next_frame_to_send = base;

}

printf("Sender: All frames transmitted successfully using Selective Repeat.\n");

}

int main() {

srand(time(NULL)); // Seed for random number generation

int total_frames;

printf("Enter the total number of frames to send: ");

scanf("%d", &total_frames);

printf("\n--- Go-Back-N Protocol ---\n");

sender_sr(total_frames);

printf("\n--- Selective Repeat Protocol ---\n");

sender_sr(total_frames);

return 0;

}

OUTPUT